How to use the Data logger Air Core v2.0

************  AIR core hardware  ************

- Place the air core on a table and open it.

- Remove the small foam and open the Royal Air cores Box. Remove the data logger out of the box; please, grab it by the battery or the metal plate. Be careful.

- Replace the two 9 Volt (pre. Lithium) batteries and check the micro-SD card is present.

- Take the computer with the Adruino software and start it.  The computer will be connected with the data logger, by a mini usb cable.

************  AIR core software  ************

- Open the Arduino software.

- Open the “SERIAL MONITOR”, on the top left.

- When it is right, the next text will be showed. “…working is loadmode. Give a command: “

- Give command, to set the time: “!TIME”

- Give command, give the time: ”2015-06-30 16:00:00”

- Give command, get the time: “?TIME” . It will replay the time, 15 times.

Give command, “QUIT”. This will stop initialization and start program.

************  AIR core automatic valve  ************

- Place all back, data logger is ready.

- Only the automatic valve has to be set.

- Or like this.

- Finished. Put the blue seal on the inlet, till lanch. Don´t forget it.

************  AIR core software problems  ************

- Check BOARD : ->Tools->Board->Arduino Nano

- Check Processor: -> Tools->Processor->ATMega328

- Check Port: ->Tools->Port